BAI Model: 909U (Flat Top)

How to order: Model + Heater Code with color code + Options (example 909U02000NW-1655-T1)

| Length In. (mm) | Density |  | Watts | Heater Code |  |  |  |  |  |  | Watts at 240/208V | Heater Code |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W/Ft | W/m |  |  | 208V | Amps |  | 277V | Amps |  |  | 240/208V | Amps |
| 27.2 (690) | 375 | 1250 | 800 |  | 00800C‡-690 | 3.8 |  | 00800N‡-690 | 2.9 |  | 800/600 | 00800V‡-690 | 3.3/2.9 |
|  | 500 | 1640 | 1000 |  | 01000C‡ | 4.8 |  | 01000N‡ | 3.6 |  | 1000/750 | 01000V‡ | 4.2/3.6 |
| 37.2 (944) | 375 | 1250 | 1000 |  | 01000C‡-944 | 4.8 |  | 01000N‡-944 | 3.6 |  | 1000/750 | 01000V‡-944 | 4.2/3.6 |
|  | 500 | 1640 | 1500 |  | 01500C‡ | 7.2 |  | 01500N‡ | 5.4 |  | 1500/1125 | 01500V‡ | 6.3/5.4 |
| 47.2 (1198) | 375 | 1250 | 1500 |  | 01500C‡-1198 | 7.2 |  | 01500N‡-1198 | 5.4 |  | 1500/1125 | 01500V‡-1198 | 6.3/5.4 |
|  | 500 | 1640 | 2000 |  | 02000C‡ | 9.6 |  | 02000N‡ | 7.2 |  | 2000/1500 | 02000V‡ | 8.3/7.2 |
| 57.2 (1452) | 375 | 1250 | 1750 |  | 01750C‡-1452 | 8.4 |  | 01750N‡-1452 | 6.3 |  | 1750/1313 | 01750V‡-1452 | 7.3/6.3 |
|  | 500 | 1640 | 2500 |  | 02500C‡ | 12.0 |  | 02500N‡ | 9.0 |  | 2500/1875 | 02500V‡ | 10.4/9.0 |
| 65.2 (1655) | 375 | 1250 | 2000 |  | 02000C‡-1655 | 9.6 |  | 02000N‡-1655 | 7.2 |  | 2000/1500 | 02000V‡-1655 | 8.3/7.2 |
|  | 500 | 1640 | 3000 |  | 03000C‡ | 14.4 |  | 03000N‡ | 10.8 |  | 3000/2250 | 03000V‡ | 12.5/10.8 |
| 75.2 (1909) | 375 | 1250 | 2500 |  | 02500C‡-1909 | 12.0 |  | 02500N‡-1909 | 9.0 |  | 2500/1875 | 02500V‡-1909 | 10.4/9.0 |
|  | 500 | 1640 | 3500 |  | 03500C‡(1) | 16.8 |  | 03500N‡ | 12.6 |  | 3500/2625 | 03500V‡ | 14.6/12.6 |
| 83.2 (2112) | 375 | 1250 | 2750 |  | 02750C‡-2112 | 13.2 |  | 02750N‡-2112 | 9.9 |  | 2750/2066 | 02750V‡-2112 | 11.5/9.9 |
|  | 500 | 1640 | 4000 |  | 04000C‡(1) | 19.2 |  | 04000N‡ | 14.4 |  | 4000/3000 | 04000V‡(1) | 16.7/14.4 |

Other single-phase voltages available, consult factory. All models listed are single-phase.

‡Standard colors - Add W for white finish or A for almond finish to the end of catalog number when ‡ is indicated.

(1) If control option R7 is required for these heaters, (Qty 2) R7 Triac proportional relays are required.

Factory Built-in Options

| Code | Description |  |
| --- | --- | --- |
| D1 | Disconnect Switch, Three-pole, 600V, 40 Amp |  |
| F | Front Air Outlet (BAI Only) |  |
| R1 | 24V Relay, 240V, 22 Amp; 277V, 19 Amp |  |
| R2 | 24V Relay with Transformer, 240V, 22 Amp; 277V, 19 Amp |  |
| R3 | 24V Double Pole Relay, 240V, 25 Amp; 480V, 12.5 Amp |  |
| R4 | 240V Relay, Single-pole, 240V, 25 Amp; 480V, 12.5 Amp |  |
| R5 | 240V Relay, Double-pole, 240V, 25 Amp; 480V, 12.5 Amp |  |
| R6 | Pneumatic Relay, 480V, 25 Amp |  |
| R7 | 24V Proportional Relay |  |
| S | Smooth Finish on Heater Back |  |
| T1 | Tamperproof Thermostat, Single-pole, 240V, 30 Amp; 480V, 15 Amp |  |
| T2 | Tamperproof Thermostat, Double-pole, 480V, 15 Amp |  |
| X1 | 240V Transformer – 50 VA |  |
| X2 | 24V Transformer – 50 VA |  |

Field-Installed Options

(shipped loose for field installation into heater)

| Part No. | Description |  |  |
| --- | --- | --- | --- |
| 909-124609¶ | BAI Corner |  |  |
| 909-124610¶ | Square Pedestal Kit | 3-4” High |  |
| 909-124611¶ | Sub Base Heater Length In. (mm) | 27.2 (690) |  |
| 909-124612¶ |  | 37.2 (944) |  |
| 909-124613¶ |  | 47.2 (1198) |  |
| 909-124614¶ |  | 57.2 (1452) |  |
| 909-124615¶ |  | 65.2 (1655) |  |
| 909-124616¶ |  | 75.2 (1909) |  |
| 909-124617¶ |  | 83.2 (2112) |  |
| 909-218276¶ | Empty Cabinet Length In. (mm) | 27.2 (690) |  |
| 909-218277 ¶ |  | 37.2 (944) |  |
| 909-218278 ¶ |  | 47.2 (1198) |  |
| 909-218279 ¶ |  | 57.2 (1452) |  |
| 909-218280 ¶ |  | 65.2 (1655) |  |
| 909-218281¶ |  | 75.2 (1909) |  |
| 909-218282 ¶ |  | 83.2 (2112) |  |
| 909-124618¶ | Recessed Trim Kit Heater Length In. (mm) (BAI Only, Option F Required) | 27.2 (690) |  |
| 909-124619¶ |  | 37.2 (944) |  |
| 909-124620¶ |  | 47.2 (1198) |  |
| 909-124621¶ |  | 57.2 (1452) |  |
| 909-124622¶ |  | 65.2 (1655) |  |
| 909-124623¶ |  | 75.2 (1909) |  |
| 909-124624¶ |  | 83.2 (2112) |  |
| 909-124628¶ | Control Box – 12” | 12 (305) |  |
| 909-124629 | 24V Relay Kit, Single-pole, 240V, 22 Amp; 277V, 19 Amp |  |  |
| 909-124630 | 24V Relay Kit, Double-pole, 240V, 25 Amp; 480V, 12.5 Amp |  |  |
| 909-124633 | 24V Relay Kit with Transformer, 208V, 22 Amp |  |  |
| 909-124634 | 24V Relay Kit with Transformer, 240V, 22 Amp |  |  |
| 909-124635 | 24V Relay Kit with Transformer, 277V, 19 Amp |  |  |
| 909-224759 | Tamperproof Single-pole Thermostat Kit |  |  |
| 909-224760 | Tamperproof Double-pole Thermostat Kit |  |  |

¶ Standard colors - Add W for white finish or A for almond finish to the end of part number.

Unit with Grille on Top

Right Side

View

Bottom View

Rear View

Unit with Grille on Front (Option F)

Front View

Architect’s and Engineers Specification

Heating Elements – Stainless steel elements with aluminum fins float on high temperature nylon bushings for quiet operation.

Housing – Cabinet is constructed of 20-gauge steel with a 16-gauge steel front cover. The unit has a polyester powder paint finish in white or almond.

Inlet/Outlet Grilles – The fresh air inlet grille is located on the front of the unit. An extruded aluminum outlet grille discourages tampering with the heating element.

Built-in Controls – Linear limit, automatic reset thermal cutout.

Installation Requirements – Field or factory-installed low voltage relay and thermostat kits. These units are not for residential use in the United States.

Rough In Dimensions – Heater Length + 1” wide x 15” high x 6” deep.