Color	Code

Bronze	BZ

Metallic Silver	AE

Metallic Charcoal	CM

Textured Black	NR

Beige-Grey	BG

Aluminum	AL

White	W

Almond	A

Built-in tamperproof thermostat is included.

(1) Total Amps = Heating element amps and motor amps.

All models listed are single-phase. Other single-phase voltages available. Consult factory.

¶Add color code to end of catalog number.

| Watts | Volts | Phase | Total Amps (1) | CFM | Approximate Air Temperature Rise oF (oC) | Weight lb (kg) | Catalog Number |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1500/1125 | 240/208 | 1 | 6.6/5.8 | 160 | 30/22 (17/12) | 24 (11) | 933U01500V |  |
| 1500 | 120 | 1 | 12.9 | 160 | 30 (17) | 24 (11) | 933U01500B |  |
| 1500 | 277 | 1 | 5.8 | 160 | 30 (17) | 24 (11) | 933U01500N |  |
|  | 480 | 1 | 3.5 | 160 | 30 (17) | 28 (13) | 933U01500T |  |
| 2000/1500 | 240/208 | 1 | 8.7/7.6 | 160 | 40/30 (22/17) | 24 (11) | 933U02000V |  |
| 2000 | 208 | 1 | 10.0 | 160 | 40 (22) | 24 (11) | 933U02000C |  |
|  | 277 | 1 | 7.6 | 160 | 40 (22) | 24 (11) | 933U02000N |  |
|  | 480 | 1 | 4.5 | 160 | 40 (22) | 28 (13) | 933U02000T |  |
| 3000/2250 | 240/208 | 1 | 12.9/11.2 | 160 | 60/45 (33/25) | 24 (11) | 933U03000V |  |
| 3000 | 208 | 3 | 8.74 | 160 | 60 (33) | 28 (13) | 933U03000D |  |
|  | 277 | 1 | 11.2 | 160 | 60 (33) | 24 (11) | 933U03000N |  |
|  | 480 | 1 | 6.7 | 160 | 60 (33) | 28 (13) | 933U03000T |  |
|  | 480 | 3 | 4.01 | 160 | 60 (33) | 32 (15) | 933U03000U |  |
| 4000/3000 | 240/208 | 1 | 17.0/14.8 | 160 | 80/60 (44/33) | 24 (11) | 933U04000V |  |
|  | 240/208 | 3 | 10.0/8.7 | 160 | 80/60 (44/33) | 28 (13) | 933U04000P |  |
| 4000 | 208 | 1 | 19.6 | 160 | 80 (44) | 24 (11) | 933U04000C |  |
|  | 208 | 3 | 11.52 | 160 | 80 (44) | 28 (13) | 933U04000D |  |
|  | 277 | 1 | 14.8 | 160 | 80 (44) | 24 (11) | 933U04000N |  |
|  | 480 | 1 | 8.7 | 160 | 80 (44) | 28 (13) | 933U04000T |  |
|  | 480 | 3 | 5.22 | 160 | 80 (44) | 32 (15) | 933U04000U |  |
| 4800/3600 | 240/208 | 1 | 20.4/17.7 | 160 | 96/72 (53/40) | 24 (11) | 933U04800V |  |
| 4800 | 208 | 1 | 23.5 | 160 | 96 (53) | 24 (11) | 933U04800C |  |
|  | 277 | 1 | 17.7 | 160 | 96 (53) | 24 (11) | 933U04800N |  |
|  | 480 | 1 | 10.4 | 160 | 96 (53) | 28 (13) | 933U04800T |  |
| 5000/3750 | 240/208 | 3 | 12.4/10.8 | 160 | 100 (55) | 28 (13) | 933U05000P |  |
| 5000 | 208 | 3 | 14.3 | 160 | 100 (55) | 28 (13) | 933U05000D |  |
|  | 480 | 3 | 0.4 | 160 | 100 (55) | 32 (15) | 933U05000U |  |
| 6000/4500 | 240/208 | 1 | 25.4/22.0 | 320 | 60/45 (33/25) | 50 (23) | 933U06000V |  |
| 6000 | 277 | 1 | 22.1 | 320 | 60 (33) | 55 (25) | 933U06000N |  |
|  | 480 | 1 | 12.9 | 320 | 60 (33) | 55 (25) | 933U06000T |  |
| 8000/6000 | 240/208 | 1 | 33.7/29.2 | 320 | 80/60 (44/33) | 50 (23) | 933U08000V |  |
| 8000 | 208 | 1 | 38.9 | 320 | 80 (44) | 50 (23) | 933U08000C |  |
|  | 277 | 1 | 29.3 | 320 | 80 (44) | 55 (25) | 933U08000N |  |
|  | 480 | 1 | 17.1 | 320 | 80 (44) | 55 (25) | 933U08000T |  |
| 12000/9000 | 240/208 | 3 (2) | 29.3/25.4 | 480 | 80 (44) | 90 (41) | 933U12000P |  |
| 12000 | 208 | 3 | 33.7 | 480 | 80 (44) | 90 (41) | 933U12000D |  |
|  | 240 | 3 | 29.3 | 480 | 80 (44) | 90 (41) | 933U12000K |  |
|  | 277 | 1 | 43.7 | 480 | 80 (44) | 90 (41) | 933U12000N |  |
|  | 480 | 1 | 25.4 | 480 | 80 (44) | 90 (41) | 933U12000T |  |
|  | 480 | 3 | 14.8 | 480 | 80 (44) | 90 (41) | 933U12000U |  |

Factory Built-in Options

| Code | Description |  |
| --- | --- | --- |
| D1 | Disconnect Switch 600V, Three-pole, 40 Amp |  |
| D3 | Disconnect Switch 277V, Doube-pole, 20Amp |  |
| R1 | 24V Relay (2) |  |
| R2 | 24V Relay with Transformer (2) |  |
| S3 | Fan On/Auto Switch |  |

Field-Installed Options (shipped loose for field Installation into heater)

| Catalog Number | Description |  |
| --- | --- | --- |
| 933-124501¶ | Surface Mounting Box (1500-4800W) |  |
| 933-158471¶ | Surface Mounting Box (6000-8000W) |  |
| 933-269345¶ | Surface Mounting Box (12000W) |  |
| 933-151114¶ | 1” Semi Recessed Trim Kit (1500-4800W) |  |
| 933-166530¶ | 1” Semi Recessed Trim Kit (6000-8000W) |  |
| 933-269346¶ | 1” Semi Recessed Trim Kit (12000W) |  |
| 933-151115¶ | 2” Semi Recessed Trim Kit (1500-4800W) |  |
| 933-166531¶ | 2” Semi Recessed Trim Kit (6000-8000W) |  |
| 933-269347¶ | 2” Semi Recessed Trim Kit (12000W) |  |

(2)  A built-in thermostat will not be supplied when option “R1” or “R2” are selected.  Contact factory if built-in thermostat is required with these options.

¶Add color code to end of catalog number.

Architect’s and Engineer’s Specifications

The specified heating equipment shall include an electric, fan-forced air wall heater suitable for space heating as manufactured by INDEECO.  The wall heater shall be designed for wall recess or surface mounting.  Heaters shall be third party approved to UL standard 2021.  CSA-US or equivalent is acceptable.

Heater Housing shall be constructed from heavy 18 gauge steel.

Grille shall have extruded aluminum louvers with a dark brown epoxy/powder paint finish.  Grille openings shall be less than 0.25” to discourage tampering with the elements (pencil-proof).  Contact factory for custom colors.

Heater Element shall be of finned tubular type.

Fan and Motor shall be totally enclosed and have permanently lubricated ball bearings.  Fan shall operate at 160 CFM minimum.

Fan Over-ride shall purge heater of residual heat to prevent an over heat condition.

Thermal Cutout shall be of the bulb and capillary type and built into the system.

Thermostat is tamperproof and included as standard.  Adjustable from 32-120⁰F.

Disconnect Switch is offered as an option and is built into the back of the system.  The disconnect switch shall provide positive disconnect of the main power source.

Low Voltage Relays with or without transformers are available built-in or supplied separately as kits.

Rough In Dimensions

- 1500/1125 - 4800 Watt heaters: 14-1/2” wide x 19-7/8” high x 4-1/2” deep.

- 6000/4500 - 8000 Watt heaters: 30-3/16” wide x 19-7/8” high x 4-1/2” deep.

- 12000/9000 Watt heaters: 46-5/8” wide x 19-7/8” high x 4-1/2” deep.