|  | Built-in digital thermostat with timer is included. Watts; Volts; Amps; Weight lb (kg); Catalog Number. 1500; 120; 12.5; 22.8 (10.3); 940U01500B¶. 2000; 240; 8.3; 22.8 (10.3); 940U02000J¶. ¶Standard colors – Add W for white finish or A for almond finish to the end of catalog number. Factory Built-in Options Code; Description. D3; Disconnect Switch, 277V, Double-pole, 20 Amp. Field-Installed Options Catalog Number; Description. 940-213895¶; Cast Aluminum Surface Mounting Box. ¶Standard colors – Add W for white finish or A for almond finish to the end of catalog number. |
| --- | --- |

Architect’s and Engineer’s Specifications

Heating Elements – Instant heat nickel-chrome coil resistant wire element.

Outlet Grille – Heavy cast aluminum front cover with steel bar inlet and outlet grilles has a polyester powder paint finish available in white or almond.

Built-in Controls – Digital display electronic thermostat with timer, automatic reset thermal cutout, temperature sensor and LED night light with on-off switch.

Fan motor – Whisper-quiet squirrel-cage fan with factory-lubricated motor.

Installation Requirements –Maintain a minimum of 4-inches (102 mm) clearance below heater, 1-inch (25 mm) to adjacent walls and 48-inches (1219 mm) in front of heater.

Rough In Dimensions – All heaters: 18-5/8” wide x 13-3/8” high x 2-3/4” deep.